| PART B FIRE SAFETY |  |
| --- | --- |
| #### Means of escape B1. The building shall be designed and constructed so that there are means of escape in case of fire from the building to a place of safety outside the building capable of being safely and effectively used at all material times. | Requirement B1 does not apply to any prison provided under section 33 of the Prisons Act 1952(12) (power to provide prisons etc.). |
| #### Internal fire spread (linings) B2. — (1) To inhibit the spread of fire within the building the internal linings shall— (a)resist the spread of flame over their surfaces; and (b)have, if ignited, a rate of heat release which is reasonable in the circumstances. (2) In this paragraph “internal linings” means the materials lining any partition, wall, ceiling or other internal structure. |  |
| #### Internal fire spread (structure) B3. — (1) The building shall be designed and constructed so that, in the event of fire, its stability will be maintained for a reasonable period. (2) A wall common to two or more buildings shall be designed and constructed so that it resists the spread of fire between those buildings. For the purposes of this sub-paragraph a house in a terrace and a semi-detached house are each to be treated as a separate building. (3) To inhibit the spread of fire within the building, it shall be sub-divided with fire-resisting construction to an extent appropriate to the size and intended use of the building. | Requirement B3(3) does not apply to material alterations to any prison provided under section 33 of the Prisons Act 1952. |
| (4) The building shall be designed and constructed so that the unseen spread of fire and smoke within concealed spaces in its structure and fabric is inhibited. |  |
| #### External fire spread B4. — (1) The external walls of the building shall resist the spread of fire over the walls and from one building to another, having regard to the height, use and position of the building. (2) The roof of the building shall resist the spread of fire over the roof and from one building to another, having regard to the use and position of the building. |  |
| #### Access and facilities for the fire service B5. — (1) The building shall be designed and constructed so as to provide facilities to assist fire fighters in the protection of life. (2) Provision shall he made within the site of the building to enable fire appliances to gain access to the building. |  |